301H-01S   DUPLICATION TRANSMITTER

# INSTRUCTIONS

## HOW TO DUPLICATE THE CODE:

- Take Jumper—J2 off to enter learning mode.

- By pressing and holding the button of 301H-01S first then pressing and holding the button of original transmitters, their position is such as sketch 1.  When the code well learn from the original TX, the LED bulb of the 301H-01S blinking  solid on  going off.

- The buttons pressing must be button 1 vis-à-vis button 1, button 2 vis-à-vis button 2, … until all buttons pressed.

- If the code learning fail, the LED bulb of the 301H-01S keep blinking when pressing the button. Please change the holding position of both transmitters to “side by side”, or “back to back” or “overlapping both TX” or any position better than head to head.

## HOW TO DUPLICATE THE FREQUENCY:

- Check the frequency range of the original transmitter, by putting the original transmitter at TX zone of the digital frequency counter and press the button of original TX. The digital screen of the frequency counter will display the frequency of the original TX in figures.

- Tune the frequency of the 301H-01S as same as the frequency of the original TX via the digital frequency counter. (Code learning has been completed first.)

- The corresponding frequency range with the Jumper—J1 position as refer following:

| - Frequency Range | - Jumper Position |
| --- | --- |
| - 260 – 350 MHz |  |
| - 300 – 460 MHz |  |

- NOTES:

- For Jumper 2:

- Jumper On  Transmission Mode.

### Jumper Off  Code Learning Mode.

- To avoid interference, keep away from mobile phone while duplicating. And don’t operate all the procedures on the metal surface.

- Sketch  1.