301H-2H  Code / Frequency Simultaneously Learning

[1]. Transmission     Frequency Range: 250-450Mhz

Pressing any one channel button, LED lights on solid。

[2]. To activate Code / Frequency Learning Mode

Press and hold buttons 1+2  together, count LED flashing

- LED Flashes once then release – Transmission mode

- LED Flashes twice then release – Learning mode

### [3]. To Proceed Code / Frequency Learning --Code Learning mode

As sketch (1) ; Hold original (Old) transmitter at Left hand side and 301H-2H at Right hand side.

a). Keep 15cm distance between Old TX (transmitter) and 301H-2H

b). Pressing button 1. of Old TX and 301H-2H together at the same time.

c). 301H-2H flashes twice and goes off,

d). Now slowly move the Old TX to right hand side and stop moving when the LED of 301H-2H is on solid.

e). Still pressing & holding 3 seconds after the LED of 301H-2H goes off then release your hand.

Code/frequency learning is complete for button 1.

f ). Button 2 or 3… please proceed the steps as  a) …e)

Remark :

[A]. If code/frequency learning of 301H-2H is not successful from the above proceeding, please change the position of Old TX as sketch (2) and starting again from steps a)…e) .

[B]. Step b.) – If LED of 301H-2H lights ON, 301H-2H is not under Code/Frequency Learning mode, please proceed

step [2] again.

[C]. A/B section switch: Turn switch up at A. section – 2 channels are able to learn 2 different codes & frequencies

Turn switch down at B section–2 channels are able to learn 2 different codes & frequencies

[D]. It will skip out the ‘Learning Mode’ if pressing any buttons over 4 times. Please renew to enter the

‘learning mode’ and do the procedures again for completing duplication.

Sketch (1).

Sketch (2).

`      SIMPLY OPERATION INSTRUCTION

Press and hold buttons 1+2  together, count LED flashing

◆LED Flashes once then release – Transmission mode

◆LED Flashes twice then release – Learning mode

a). Keep 15cm distance between Old TX (transmitter) and 301H-2H

b). Pressing button 1. of Old TX and 301H-2H together at the same time.

c). 301H-2H flashes twice and goes off,

d). Now slowly move the Old TX to right hand side and stop moving when the LED of 301H-2H is on solid.

e). Still pressing & holding 3 seconds after the LED of 301H-2H goes off then release your hand.

Code/frequency learning is complete for button 1.

f ). Button 2 or 3… please proceed the steps as  a) …e)