Learning Low Frequency

301H-2L                  25-40Mhz   P1/2

# INSTRUCTION

Adjusting Frequency:

- Open the case of the original transmitter and be aware of its frequency on the quartz component indication.

- Frequency Jumper: Select one of the frequency jumper(F1/F2/F3) and correspond the quartz (Q1/Q2/Q3) to plug jumper into the position.

(Q1/Q2: Indicated quartz built when ex-factory ; Q3: Movable quartz) (refer sketch 1.)

- Frequency Range Jumper: Select the corresponding frequency range and plug the jumper into.

- Adjusting to the best transmission point for the frequency:

1. Take the ‘Learning Jumper’ to the position A. (as Sketch 1.)

2. Press and hold the button of K2, then slowly tune the V (variable capacity)

(as Sketch 1.) via tuner until the LED of 301H-2L to the dimmest or going off.

Learning Code:

- Take the ‘Learning Jumper’ to the position B.(as Sketch 1)

- Put the original transmitter close to the side of 301H-2L as Sketch 2.

- Press and hold the both transmitters to adjust both opposite position by the LED of 301H-2L presenting the brightest, it’s the best position for learning. Release two hands.

- Then press and hold the button K1, the LED of 301H-2L is fast and blinking twice. It means the 301H-2L enters the code learning mode, but don’t release K1.

- At the same time, press and hold the button of original transmitter. If the LED of 301H-2L bright and slowly blinks 4 times then release both buttons, it has been completely copied for the code of K1.

- According to the above steps to copy other keys.

- Take the ‘Learning Jumper’ to the position C (as Sketch 1) and cover the outer-case   when the whole buttons have been copied completely.

Transmission Mode:

- ‘Learning Jumper’ has to be on the position C (as Sketch 1).

When press K1 or K2 and the LED of 301H-2L is solid on, the 301H-2L is in transmission.

Remarks:

▲The frequency jumper of F1/F2/F3(as Sketch 1) of selection can be corresponding the quartz component of the PCB---Q1/Q2/Q3 which has been shown each frequency on.

(Custom-made: Various frequencies are available made on request, frequency range at 25Mhz - 40Mhz, at most 3 kinds of frequency).

- Available buttons are [K1] 、 [K2] 、 [K1+K2] 。

- Please pay attention the frequency range jumper whether is in right position.

- As ‘Adjusting to the best transmission point’, please quickly complete the action for saving the life of battery power.

## P2/2