[image: image1.png]. 15555
v @l || =
A.STX-501H HEAFEHEER,

B. 3% T /A S HEAEER B

REMOTE || C.EF{7ABZESTX-501Hig
TR BAY), B | ST
@)

BIREAIRF SRR
BEFEEEE) BEFAE




[image: image6.png]HTIE

famg
R

L0BHER



            301H-5H8  CODE / Frequency Learning Transmitter
[1]   Transmission :
Pressing any one channel button, LED lights on solid。
[2] To activate Code / Frequency Learning Mode:     

Press and hold buttons 1+2  together, count LED flashing

· LED Flashes once then release – Transmission mode
· LED Flashes twice then release – Code Learning mode
[3] To Proceed Code / Frequency Learning –CODE LEARNING MODE
As sketch(1) ; Hold original (Old) transmitter at Left hand side and

301H-5H8 at Right hand side

a) Keep 15cm distance between Old TX (transmitter) and 301H-5H8
b) Pressing button 1. of Old TX and 301H-5H8 together at the same time

c) 301H-5H8 flashes twice and goes off

d) Now slowly move the Old TX to right hand side and stop moving when the LED of 301H-5H8 is lighting on solid
e) Still pressing & holding 3 seconds after the LED of 301H-5H8 goes off then release your both hands.

Now the code/frequency learning is complete for button 1.

f) button 2 or 3… please proceed the steps as  a) …e)

Remark :
[A]. If code/frequency learning of 301H-5H8 is not successful from the above proceeding, please change the position of Old TX as sketch (2) and starting again from steps a)…. e)
[B]. Step b.) – If LED of 301H-5H8 lights ON, 301H-5H8 is not under Code/Frequency Learning mode,  please proceed step [2]  again.

[C]. A/B section switch: 

Turn switch up at A. section – 4 channels are able to learn 4 different codes with 4 different frequencies

Turn switch down at B section–4 channels are able to learn 4 different codes with 4 different frequencies

[D]. It will skip out the “Learning Mode” if pressing any buttons over 8 times. Please renew to enter the

learning mode and do the copying procedures again for completing duplication.


[image: image7.png]HTIE

famg
R

L0BHER




[image: image2.png]Sketch 2.

Left hand side move to —s
Right hand side (Still)




	Simply Learning Operation

	[image: image3.jpg]



	Step 1. 

· Press and hold buttons[1+2] together, count LED flashing. 

· LED Flashes twice then release 

	[image: image4.jpg]



	Step 2. 

a. Keep 15cm distance between Old TX and 301H-5H 

b. Pressing button 1. of Old TX and 301H-5H together at the same time 

c. 301H-5H flashes twice and goes off 

	[image: image5.jpg]



	Step 3. 

d. Now slowly move the Old TX to right hand side and stop when 301H-5H LED is lighting on solid 

e. 2 seconds after, LED goes off, code/frequency learning is complete for button 1 

f. button 2 or 3... please proceed the steps as a...e 


� EMBED PBrush  ���





switch





Section





Channels button





LED





Sketch 1.





15cm








Right hand side (Still)





Left hand side moving to 





Original



































_1056969988

_1057043051

