301H-5H8  CODE / Frequency Learning Transmitter

Section

Channels button

LED

switch

[1]   Transmission :

Pressing any one channel button, LED lights on solid。

[2] To activate Code / Frequency Learning Mode:

Press and hold buttons 1+2  together, count LED flashing

- LED Flashes once then release – Transmission mode

- LED Flashes twice then release – Code Learning mode

[3] To Proceed Code / Frequency Learning –CODE LEARNING MODE

As sketch(1) ; Hold original (Old) transmitter at Left hand side and

301H-5H8 at Right hand side

a) Keep 15cm distance between Old TX (transmitter) and 301H-5H8

b) Pressing button 1. of Old TX and 301H-5H8 together at the same time

c) 301H-5H8 flashes twice and goes off

d) Now slowly move the Old TX to right hand side and stop moving when the LED of 301H-5H8 is lighting on solid

e) Still pressing & holding 3 seconds after the LED of 301H-5H8 goes off then release your both hands.

Now the code/frequency learning is complete for button 1.

f) button 2 or 3… please proceed the steps as  a) …e)

Remark :

[A]. If code/frequency learning of 301H-5H8 is not successful from the above proceeding, please change the position of Old TX as sketch (2) and starting again from steps a)…. e)

[B]. Step b.) – If LED of 301H-5H8 lights ON, 301H-5H8 is not under Code/Frequency Learning mode,  please proceed step [2]  again.

[C]. A/B section switch:

Turn switch up at A. section – 4 channels are able to learn 4 different codes with 4 different frequencies

Turn switch down at B section–4 channels are able to learn 4 different codes with 4 different frequencies

[D]. It will skip out the “Learning Mode” if pressing any buttons over 8 times. Please renew to enter the

learning mode and do the copying procedures again for completing duplication.

Sketch 1.

15cm

Left hand side moving to

Right hand side (Still)

Original

| Simply Learning Operation |  |
| --- | --- |
|  | Step 1. - Press and hold buttons[1+2] together, count LED flashing. - LED Flashes twice then release |
|  | Step 2. - Keep 15cm distance between Old TX and 301H-5H - Pressing button 1. of Old TX and 301H-5H together at the same time - 301H-5H flashes twice and goes off |
|  | Step 3. - Now slowly move the Old TX to right hand side and stop when 301H-5H LED is lighting on solid - 2 seconds after, LED goes off, code/frequency learning is complete for button 1 - button 2 or 3... please proceed the steps as a...e |