301H-5HS   Code / Frequency Simultaneously Learning

### Frequency Range:  LC: 250-450Mhz;  SAW: 433.92Mhz

[1]. Transmission

Pressing any one channel button, LED lights on solid。

LED

[2]. To activate Code / Frequency Learning Mode

Press and hold buttons 1+2  together, count LED flashing

Channels

button

- LED Flashes once then release – Transmission mode

- LED Flashes twice then release – LC Code Learning mode

- LED Flashes thrice then release –SAW Code Learning mode

[3]. To Proceed Code / Frequency Learning – Code Learning mode

As sketch (1) ; Hold original (Old) transmitter at Left hand side and

301H-5HS at Right hand side.

a). Keep 15cm distance between Old TX (transmitter) and 301H-5HS

b). Pressing button 1. of Old TX and 301H-5HS together at the same time

c). 301H-5HS flashes twice and goes off

d). Now slowly move the Old TX to right hand side and stop when 301H-5HS LED is

lighting on solid

e). 3 seconds after, LED goes off, code/frequency learning is complete for button 1.

f). Button 2 or 3… please proceed the steps as  a) …e)

Remark :

[A]. If code/frequency learning of 301H-5HS is not successful from the above proceeding, please change the position of Old TX as sketch (2) and starting again from steps a)…. e) .

[B]. Step b.) – If LED of 301H-5HS lights ON, 301H-5HS is not under Code/Frequency Learning mode, please proceed step [2] again.

Sketch (1).

Sketch (2).

Original

Original

15cm

15cm

Left hand side

moving to

Right hand side (Still)

Left hand side moving to

Right hand side (Still)